Galena Action Ring

System Architecture & Development Documentation

Version 1.0 — June 2026

# Table of Contents

1.  Project Overview

2.  System Architecture

3.  Hardware Components

4.  ESP-NOW Communication Protocol

5.  USB HID Protocol

6.  WinUI 3 Desktop Application

6.1  Service Layer

6.2  Model Layer

6.3  MainWindow Architecture

6.4  On-Screen Display (OSD)

7.  ESP32-S3 Action Ring Firmware

7.1  Module Map

7.2  Task Architecture

7.3  Shared Globals

7.4  Key Source Files

8.  ESP32-C3 Light Bar Firmware

8.1  Module Map

8.2  Task Architecture

8.3  Key Source Files

9.  Data Flow Scenarios

9.1  Encoder Turn → Brightness Change

9.2  Button Press → Light Toggle

9.3  Boot Sequence & Sync

9.4  OSD Request/Response Cycle

10. Code Professionalization History

11. Build & Deployment

# 1. Project Overview

Galena Action Ring is a three-component hardware/software system that provides a physical rotary encoder ring and button for controlling PC functions such as volume, brightness, media playback, application launching, and monitor light control.

The system consists of three codebases that communicate over two channels: USB HID (for PC communication) and ESP-NOW (for wireless device-to-device communication):

Desktop App (C#/.NET 8): WinUI 3 Desktop Application

Action Ring: ESP32-S3 Action Ring Firmware (C/ESP-IDF):

Light Bar: ESP32-C3 Light Bar Firmware (C/ESP-IDF):

# 2. System Architecture

The system follows a hub-and-spoke topology. The Action Ring serves as the central hub connected to the PC via USB, while the Light Bar communicates wirelessly with the Action Ring over ESP-NOW.

Communication channels:

Channel 1 — USB HID (Vendor-Defined, Usage Page 0xFF00):

PC ↔ Action Ring. The Action Ring appears as a custom HID device. The app reads 8-byte input reports and writes 1-byte feature reports for OSD state. Event types include brightness, encoder, button, OSD request/ack, and light state.

Channel 2 — ESP-NOW (2.4 GHz, Connectionless):

Action Ring ↔ Light Bar. A lightweight 8-byte packet carries type, value, OSD visibility flag, and USB connection state. The protocol is defined in galena_protocol.h and shared between both firmwares.

System Flow Diagram:

┌─────────────────┐    USB HID     ┌──────────────────┐   ESP-NOW    ┌──────────────────┐

│   WinUI 3 App   │◄──────────────►│  ESP32-S3        │◄────────────►│  ESP32-C3        │

│  (C#, .NET 8)   │  8-byte reports│  Action Ring     │  8-byte pkt  │  Light Bar       │

│                 │  1-byte feature│                  │              │  (PWM LED Strip) │

│  HidService     │                │  8 FreeRTOS tasks│              │  3 FreeRTOS tasks│

│  OsdService     │                │  USB CDC for log │              │  NVS persistence │

│  ProfileManager │                │  Encoder + Button│              │  LEDC PWM driver │

│  CanvasEditor   │                │  ESP-NOW relay   │              │                  │

└─────────────────┘                └──────────────────┘              └──────────────────┘

# 3. Hardware Components

## 3.1 ESP32-S3 Action Ring

Microcontroller: ESP32-S3 (Xtensa LX7 dual-core)

Interface: USB OTG (Full Speed) via internal PHY

Input: Rotary encoder (GPIO 11, 12) + Push button (GPIO 13)

Wireless: Wi-Fi 802.11 b/g/n (ESP-NOW protocol)

Debug: USB CDC serial over TinyUSB

## 3.2 ESP32-C3 Light Bar

Microcontroller: ESP32-C3 (RISC-V single-core)

Output: LEDC PWM driving an LED strip

Wireless: Wi-Fi 802.11 b/g/n (ESP-NOW protocol)

Storage: NVS flash for brightness/state persistence

Power: 5V USB or external

## 3.3 PC (Desktop Application)

Platform: Windows 10+ (version 19041+)

Runtime: .NET 8, Windows App SDK 1.6

Framework: WinUI 3 (Win32 desktop, not UWP)

Display: On-Screen Display overlay window

# 4. ESP-NOW Communication Protocol

## 4.1 Packet Format (galena_protocol.h)

Both firmwares share a common header file galena_protocol.h that defines the packet structure, MAC addresses, and packet type identifiers.

Packet Structure (8 bytes, packed):

typedef struct __attribute__((packed)) {

uint8_t  type;      // PKT_* constant

int32_t  value;     // payload value

uint8_t  osd_show;  // 1=OSD showing on PC, 0=hidden

uint8_t  connected; // 1=USB connected to PC, 0=standalone

} galena_packet_t;

## 4.2 Packet Types

PKT_BRIGHTNESS (0x01):

Direction: Light Bar → Action Ring. Value = brightness × 100 (0–100). Sent on encoder turn, light toggle, boot, and sync retry.

PKT_ENCODER (0x02):

Direction: Action Ring → Light Bar. Value = encoder delta (+1 or -1). Sent on every debounced encoder step.

PKT_BUTTON (0x03):

Direction: Action Ring → Light Bar. Value = 0 (tap/release), 1 (press), 2 (hold). Sent on button state transitions.

PKT_HEARTBEAT (0x04):

Direction: Action Ring → Light Bar. Value = 0. Sent every 10 seconds as a connection keep-alive ping.

PKT_LIGHT_STATE (0x05):

Direction: Light Bar → Action Ring. Value = 0 (off) or 1 (on). Sent on light toggle, boot, and sync retry.

PKT_BOOT (0x06):

Direction: Light Bar → Action Ring. Value = 0. Sent once at startup. The Action Ring suppresses relay-to-HID for 2 seconds to avoid stale-state echo.

## 4.3 MAC Address Table

Device          MAC Address (hex)

─────────────── ───────────────────

Action Ring     E0:72:A1:E9:39:44

Light Bar       88:56:A6:2A:43:B0

# 5. USB HID Protocol

## 5.1 HID Report Descriptor (galena_hid.h)

The Action Ring presents a vendor-defined HID device (Usage Page 0xFF00, Usage 0x01) with an 8-byte input report and a 1-byte feature (output) report.

HID Report Descriptor:

USAGE_PAGE (Vendor Defined 0xFF00)

USAGE (Vendor Usage 0x01)

COLLECTION (Application)

LOGICAL_MINIMUM (0)

LOGICAL_MAXIMUM (255)

REPORT_SIZE (8)      // 8 bits per field

REPORT_COUNT (8)      // 8 fields = 8 bytes

USAGE (Input Data)

INPUT (Data,Var,Abs) // Input report: 8 bytes

REPORT_COUNT (1)

USAGE (Output Data)

FEATURE (Data,Var,Abs) // Feature report: 1 byte

END_COLLECTION

## 5.2 Input Report Format

Offset  Size  Field         Description

──────  ────  ────────────  ─────────────────────────────

0       1     report_id     Always 0x00

1       1     event_type    HID_EVT_* constant (1-6)

2       1     event_value   Signed int8 payload

3-7     5     _reserved     Zero-padded (6 bytes)

## 5.3 HID Event Types

HID_EVT_BRIGHTNESS (1): Action Ring → PC. Brightness percentage (0–100).

HID_EVT_ENCODER (2): Action Ring → PC. Encoder delta (+1 or -1).

HID_EVT_BUTTON (3): Action Ring → PC. Button state (0=tap, 1=press, 2=hold).

HID_EVT_OSD_REQUEST (4): Action Ring → PC. Request OSD visibility state.

HID_EVT_OSD_ACK (5): Action Ring → PC. OSD opened/closed acknowledgment.

HID_EVT_LIGHT_STATE (6): Action Ring → PC. Light bar on/off state (1/0).

## 5.4 Feature (Output) Report

The 1-byte feature report is sent from the PC to the Action Ring to communicate the OSD visibility state: 0x00 = OSD hidden, 0x01 = OSD visible. The Action Ring includes this in the osd_show field of outgoing ESP-NOW packets.

# 6. WinUI 3 Desktop Application

The desktop app is a WinUI 3 desktop application targeting Windows 10. It manages device discovery, HID communication, profile editing, and the On-Screen Display.

## 6.1 Project Structure

Galena Action Ring/

├── App.xaml / App.xaml.cs           Application entry point

├── MainWindow.xaml / .xaml.cs       Main window (791 lines after refactor)

├── OsdWindow.xaml / .xaml.cs        On-Screen Display overlay window

├── TestOsdWindow.xaml / .xaml.cs    OSD testing window

├── Converters.cs                    XAML value converters

├── Models/

│   ├── RingProfile.cs               Profile data model

│   ├── RingNode.cs                  Action node data model

│   ├── ActionTypeItem.cs            ComboBox item model

│   └── RadialOption.cs              Radial menu record

├── Services/

│   ├── HidService.cs                HID device discovery & I/O

│   ├── OsdService.cs                OSD window management & action execution

│   ├── ProfileService.cs            JSON file-based profile persistence

│   ├── ProfileManager.cs            Profile CRUD orchestration

│   ├── CanvasEditor.cs              Canvas rendering, pointer, property editing

│   ├── NativeMethods.cs             Win32 P/Invoke declarations

│   ├── AudioVolumeControl.cs        System volume via CoreAudio API

│   ├── MaterialIcons.cs             Material Symbols icon library

│   ├── BootstrapInitializer.cs      First-run setup

│   ├── StartupService.cs            Windows startup registration

│   ├── TrayService.cs               System tray icon

│   ├── TransparentBackdrop.cs       Acrylic backdrop for OSD

│   └── UnpackagedStartupManager.cs  Unpackaged app lifecycle

└── Properties/                      Assembly metadata

## 6.2 Service Layer

HidService — Manages USB HID device lifecycle via Windows.Devices.HumanInterfaceDevice API. Uses DeviceWatcher for hot-plug detection. Exposes events for brightness, encoder, button, OSD ack, and light state. Reads input reports in a background thread loop with CancelIoEx for clean disconnect. Implements IDisposable.

ProfileService — Static CRUD service using JSON serialization to %LOCALAPPDATA%/Galena/profiles/. Provides ListProfiles(), LoadProfile(), SaveProfile(), DeleteProfile(), CreateDefault(). Each profile is a .json file named after the profile.

ProfileManager — Extracted from MainWindow during the Phase 3 refactor. Manages the in-memory profile list, selection index, editing copy, and CRUD operations. Exposes ProfilesChanged and ProfileLoaded events for MainWindow to update the UI. Encapsulates profile initialization with auto-repair of corrupted profiles and glyph migration.

CanvasEditor — Extracted from MainWindow during Phase 3. Owns all canvas rendering (radial node layout using polar-to-cartesian conversion), pointer event handling (hit testing, drag-to-reorder), node property editing, icon gallery, color picker with animation, and sub-menu navigation. Takes a CanvasEditorDependencies record with all XAML element references. Communicates with ProfileManager for save operations.

OsdService — Singleton service managing the OSD overlay window. Implements a radial action selector for Group-type nodes and direct action execution for Individual-type nodes. Handles WMI-based brightness control (with incremental 10% steps after Phase 4 fix), CoreAudio volume control, media playback commands, and system toggles. Uses DispatcherTimers for auto-hide and live state monitoring.

NativeMethods — Contains all Win32 P/Invoke declarations: CreateFileW, ReadFile, CancelIoEx, HidD_SetFeature, HidD_GetPreparsedData, and window management APIs (SetWindowPos, SetWindowLongPtr, DwmSetWindowAttribute, etc.). Also provides wrappers like OpenHidDeviceRead(), ReadInputReport(), and SendHidFeatureReport().

## 6.3 Model Layer

RingProfile — Contains Name, ProcessName (for per-process profiles), Radius, Nodes (List<RingNode>), PrimaryColor, SecondaryColor. Supports DeepCopy().

RingNode — Implements INotifyPropertyChanged. Properties: Glyph (Material Symbols character), Label, ActionType (enum), ActionData (URL or app path), Category (Individual/Group/Folder), Children (List<RingNode>? for Folder sub-menus). Supports DeepCopy().

ActionType — Enum with 18 values: None, LaunchApp, OpenUrl, VolumeUp/Down, MuteToggle, BrightnessUp/Down, MediaPlayPause/Next/Previous/SeekForward/SeekBackward, TextExpansion, Folder, CloseOsd, ToggleNightLight, VolumeControl, BrightnessControl.

## 6.4 MainWindow Architecture

MainWindow has been significantly refactored from 1475 to 791 lines. It now serves primarily as the composition root, wiring ProfileManager and CanvasEditor together:

Device discovery: HID device watcher, connection/disconnection, debug log

Navigation: NavigationView-based tabs (Bluetooth/Configure/Settings)

Profile management: Delegates to ProfileManager via thin event handlers

Canvas editing: Delegates to CanvasEditor via thin pointer/event delegates

App lifecycle: System tray, close-to-tray, reset, first-run dialog

## 6.5 On-Screen Display (OSD)

The OSD is a separate transparent overlay window (OsdWindow.xaml) that appears when the user interacts with the Action Ring. It displays radial action icons and executes actions when tapped. Key behaviors:

Radial selector for VolumeControl and BrightnessControl (Group type)

Direct execution for Individual type actions (volume step, media, launch, etc.)

Auto-hide after configurable timeout (5 seconds default)

Live state monitoring for mute and night-light toggles

Brightness preview on encoder turn when OSD is hidden

# 7. ESP32-S3 Action Ring Firmware

## 7.1 Module Map

main/

├── main.c               Entry point, GPIO init, USB init, task creation

├── usb_desc.c/.h        USB device descriptors (CDC + HID + IAD)

├── cdc_log.c/.h         CDC-ACM serial logging helper

├── hid.c/.h             USB HID send path via TinyUSB

├── espnow.c/.h          ESP-NOW init, send/receive, tasks (heartbeat, OSD poll, stats)

├── encoder.c/.h         Rotary encoder ISR + debounced event dispatch

├── button.c/.h          Push button debounce + long-press detection

├── galena_hid.h         HID report descriptor, event type enums

├── galena_protocol.h    ESP-NOW packet structure, packet types, MAC addresses

└── shared.h             Global type definitions, extern declarations, pin mappings

## 7.2 Task Architecture

The firmware runs 8 FreeRTOS tasks:

Task             Stack  Priority  Description

───────────────  ─────  ────────  ───────────────────────────────────────

usb_task         2048   6         Polls TinyUSB (tud_task), 1ms delay

espnow_task      4096   6         Receives ESOP-NOW packets from queue

enc_task         2048   4         Encoder ISR + debounce, sends to espnow

btn_task         2048   4         Button ISR + long-press, sends to espnow

hid_tx_task      2048   3         Dequeues HID events, sends via USB HID

hb_task          2048   2         Heartbeat PKT_HEARTBEAT every 10s

osd_poll_task    2048   3         OSD request via HID every 5s

espnow_stats     2048   2         WiFi channel + send/recv stats every 10s

## 7.3 Shared Globals (shared.h + main.c)

// Global state

float            g_shadow_brightness;     // Last known brightness value

SemaphoreHandle_t s_bri_mutex;            // Protects g_shadow_brightness

uint8_t          g_osd_mode;              // 0=standalone, 1=OSD controlled

// Queues

QueueHandle_t    s_espnow_queue;          // ESP-NOW receive events (10 items)

QueueHandle_t    s_hid_queue;             // HID transmit events (20 items)

// Stats (volatile — written from ISR)

volatile int32_t g_send_ok;               // Successful ESP-NOW sends

volatile int32_t g_send_fail;             // Failed ESP-NOW sends

volatile uint32_t g_rx_count;             // ESP-NOW packets received

volatile uint32_t g_osd_report_count;     // OSD HID reports sent

// GPIO pins

PIN_ENC_A        GPIO_NUM_11             // Encoder channel A

PIN_ENC_B        GPIO_NUM_12             // Encoder channel B

PIN_BUTTON       GPIO_NUM_13             // Push button

## 7.4 Key Source Files

main.c — Entry point app_main() initializes NVS, GPIO, USB PHY (OTG/internal), creates queues and mutex with NULL checks (Phase 2), configures Wi-Fi and ESP-NOW, spawns 8 tasks.

espnow.c — Implements WiFi initialization (channel 1, STA mode, PS_NONE), ESP-NOW initialization with PMK key and peer registration, send_packet() helper (Phase 4 consolidation) that builds the packet with osd_show/connected flags from USB mount state, and send callbacks for stats. The espnow_task receives packets via queue and dispatches by type: PKT_BOOT suppresses relay for 2s, PKT_BRIGHTNESS updates shadow brightness and sends HID report, PKT_LIGHT_STATE sends HID event.

hid.c — Provides hid_send_event(type, value) that queues galena_hid_report_t structures to s_hid_queue. The hid_tx_task dequeues and sends via tud_hid_report(). Handles tud_hid_get_report_cb() for brightness GET_REPORT using the mutex-protected g_shadow_brightness. Handles tud_hid_set_report_cb() for OSD feature report (1 byte).

encoder.c — Sets up GPIO interrupts on PIN_ENC_A falling edge. In the ISR, reads PIN_ENC_B to determine direction (+1 or -1) and posts to a queue with debounce (50ms window, Phase 1 fix). The encoder_task dequeues and calls espnow_send_encoder() and also passes the event through to HID.

button.c — GPIO interrupt on PIN_BUTTON (falling/rising edge). Debounces button state with 50ms timer. Detects long-press (>600ms) and reports state 2 (hold). On release reports 0 (tap). Calls espnow_send_button() and relays to HID.

# 8. ESP32-C3 Light Bar Firmware

## 8.1 Module Map

main/

├── main.c               Entry point, init sequence, task creation

├── ledc.c/.h            LEDC PWM driver for LED strip

├── nvs_driver.c/.h      NVS flash read/write for brightness + light state

├── espnow.c/.h          ESP-NOW init, send/receive, sync retry, stats

└── galena_protocol.h    ESP-NOW packet structure (shared, same file)

## 8.2 Task Architecture

The firmware runs 3 FreeRTOS tasks:

Task             Stack  Priority  Description

───────────────  ─────  ────────  ───────────────────────────────────────

espnow_task      4096   5         Receives ESP-NOW, dispatches by type

espnow_sync      2048   2         Sync retry every 3s if no ACK

espnow_stats     2048   2         Periodic stats logging every 10s

## 8.3 Key Source Files

main.c — Entry point app_main() initializes NVS (via nvs_init()), LEDC PWM (via ledc_init()), applies initial light state (apply_light()), creates ESP-NOW queue with NULL check, inits WiFi and ESP-NOW, spawns 3 tasks, and sends initial boot notification + light state + brightness to the Action Ring.

ledc.c — Configuration of LEDC timer and channel for PWM output. Implements set_brightness(float) that maps 0.0–1.0 to LEDC duty cycle (0–8191 for 13-bit). apply_light() checks g_light_on: if on, sets brightness from g_brightness; if off, sets duty to 0.

nvs_driver.c — Implements nvs_init() to initialize NVS flash. nvs_save_brightness() writes the current brightness percentage to NVS with debounce (only writes when change >= 1% threshold, Phase 2 fix). nvs_save_light_on() writes the light on/off state. All NVS calls check return values with ESP_LOGW on failure (Phase 1 fix).

espnow.c — Contains WiFi init, ESP-NOW init, send_packet() helper (Phase 4 consolidation), and three task functions. espnow_task receives PKT_BRIGHTNESS (sets absolute brightness), PKT_ENCODER (adjusts brightness by delta step), and PKT_BUTTON with value==2 (toggles light on/off and sends acknowledgement). espnow_sync_task retries light state and brightness every 3 seconds if send stats show failures. Includes MAX_ENCODER_DELTA=10 guard with BRIGHTNESS_STEP_F=0.03f per encoder tick.

# 9. Data Flow Scenarios

## 9.1 Encoder Turn → Brightness Change + OSD Preview

User turns encoder

│

▼

encoder.c ISR: reads PIN_ENC_A/B, determines delta (+1/-1)

→ posts to encoder queue (50ms debounce)

│

▼

encoder.c task: dequeues delta

→ calls espnow_send_encoder(delta) in espnow.c

→ calls send_packet(PKT_ENCODER, delta)

→ alsod calls hid_send_event(HID_EVT_ENCODER, delta)

│

├──────────────────────────┐

▼                          ▼

ESP-NOW to Light Bar      USB HID to PC

espnow_task receives      HidService.HidReadLoop

→ adjusts g_brightness    → dispatches EncoderReceived event

→ calls set_brightness()  → if OSD hidden: show brightness preview

→ calls espnow_send_      → if OSD visible: select next/prev

brightness() as ACK

## 9.2 Button Press → Light Toggle + OSD

User presses button (600ms+ hold)

│

▼

button.c: detects long-press → state=2

→ espnow_send_button(2)

│

▼

ESP-NOW to Light Bar

espnow_task: PKT_BUTTON value==2

→ toggles g_light_on

→ applies light (PWM on/off)

→ saves to NVS

→ sends PKT_LIGHT_STATE + PKT_BRIGHTNESS back

│

▼

Action Ring receives light_state

→ sends HID_EVT_LIGHT_STATE to PC

│

▼

App toggles _lightBarOn flag

→ shows brightness preview with new state

## 9.3 Boot Sequence & Sync

Light Bar powers on

│

▼

app_main():

→ nvs_init()           // restore saved brightness + state

→ ledc_init()          // apply to hardware

→ wifi_init() + espnow_init()

→ espnow_send_boot()             // PKT_BOOT → Action Ring

→ espnow_send_light_state()      // PKT_LIGHT_STATE

→ espnow_send_brightness()       // PKT_BRIGHTNESS

│

▼

Action Ring receives PKT_BOOT

→ sets s_suppress_relay_until = now + 2s

→ ignores PKT_BRIGHTNESS/LIGHT_STATE during suppression

→ prevents stale-state echo to PC

│

▼

Every 3s, Light Bar sync task:

if (g_send_fail > g_send_ok) → resend state + brightness

## 9.4 OSD Request/Response Cycle

Action Ring osd_poll_task (every 5s)

→ hid_send_event(HID_EVT_OSD_REQUEST, 0)

│

▼ (USB HID)

App HidService receives event type 4

→ calls SendOsdState(osdByte)

→ writes 1-byte feature report:

0x01 if OSD visible, 0x00 if hidden

│

▼ (USB Feature Report)

Action Ring tud_hid_set_report_cb()

→ stores in g_osd_mode (0 or 1)

→ included in next ESP-NOW packet's osd_show field

│

▼

Light Bar checks osd_show:

if osd_show=1 AND connected=1:

→ suppresses encoder-based brightness change

→ OSD on PC has focus, don't interfere

# 10. Code Professionalization History

The following is a summary of all refactoring phases applied to the three codebases during the June 2026 code review and professionalization effort:

## Phase 1 — Critical Bug Fixes

Light Bar: Removed merge conflict markers from sdkconfig.defaults

Light Bar: Added PKT_BRIGHTNESS handler in espnow_task switch/case

Light Bar: Added NVS return value checks with ESP_LOGW on failure

App: Added CancelIoEx call before closing handles in DisconnectHidDevice()

Action Ring: Protected g_shadow_brightness with mutex (data race fix)

## Phase 2 — Stability & Robustness

App: Added volatile to _lastBrightness and _lightBarOn (memory visibility)

Both firmwares: Added queue creation NULL checks with abort()

Both firmwares: Logged esp_now_send() return values at ESP_LOGW on failure

App: Replaced empty catch {} with Debug.WriteLine in MainWindow, OsdService, AudioVolumeControl

App: Fixed async void timer callback (ApplyToggleStatesToOsd → async Task)

App: Changed UpdateMediaPlayPauseIcon from async void to async Task

Light Bar: Added NVS write debounce (≥1% threshold) to prevent flash wear

## Phase 3 — Architecture Cleanup

Light Bar: Split main.c into ledc.c/.h, nvs_driver.c/.h, espnow.c/.h

Action Ring: Split main.c into usb_desc.c/.h, cdc_log.c/.h, hid.c/.h, espnow.c/.h, encoder.c/.h, button.c/.h, shared.h

App: Created HidService.cs for HID device lifecycle (extracted from MainWindow)

App: Created ProfileManager.cs for profile CRUD (extracted from MainWindow)

App: Created CanvasEditor.cs for canvas rendering/editing (extracted from MainWindow)

MainWindow reduced from 1475 to 791 lines

## Phase 4 — Polish

App: Renamed namespace Galena_Action_Ring → GalenaActionRing across all .cs files

App: Updated XAML x:Class attributes to match new namespace

Both firmwares: Consolidated 3 send functions into single send_packet() helper

App: Capped debug log to 1000 lines to prevent UI lag

Light Bar: Removed unused esp_timer dependency from CMakeLists.txt

App: Fixed SetBrightness() to query current brightness and step by 10%

# 11. Build & Deployment

## 11.1 Desktop Application

SDK: .NET 8 SDK, Windows App SDK 1.6, WinUI 3

Build command:

dotnet build "Galena Action Ring.slnx" /p:Platform=x64

Dependencies (NuGet):

Microsoft.WindowsAppSDK 2.2.0

Microsoft.Windows.SDK.BuildTools 10.0.26100.4654

System.Management 10.0.0 (WMI brightness control)

System.IO.Ports 10.0.9

TaskScheduler 2.12.12 (startup registration)

WinUIEx 2.9.1 (window management)

## 11.2 Action Ring Firmware

SDK: ESP-IDF v5.x with ESP32-S3 target

USB: TinyUSB submodule (espressif/esp_tinyusb)

Build command:

idf.py set-target esp32s3

idf.py build

idf.py flash monitor

## 11.3 Light Bar Firmware

SDK: ESP-IDF v5.x with ESP32-C3 target

Build command:

idf.py set-target esp32c3

idf.py build

idf.py flash monitor

## 11.4 Repository Structure

GitHub Repositories:

├── github.com/AkashPanja/Galena.git              → Desktop App (WinUI 3)

├── github.com/AkashPanja/GalenaActionRing-ESP32S3.git → Action Ring firmware

└── github.com/AkashPanja/GalenaLightBar-ESP32C3.git   → Light Bar firmware

All three are independent repos under the AkashPanja GitHub account.